Firehatch Temperature (EI)

Options are shown in italics

Producer:

- Hoefnagels Fire Safety

Type:

- Fire resistant hatch, Firehatch T – EI1120

Dimensions:

- Opening width (mm):	According to drawing (max. 1764 mm)

- Opening height (mm):	According to drawing (max. 1303 mm)

Versions:

- Self-closing in case of fire detection or loss of power.

Fire resistance:

- 120 minutes EI1 / EI2 (in accordance with EN 1366-7)

Installation:

- On the wall

Hatch:

- Material: Promatect, a stone like material enforced with glass fibre

- Thickness: 70 mm (with 200 mm edging)

- Colour: white

Motor housing:

- Material: Bend steel

- Thickness: 4 mm

- Colour:  Sendzimir or RAL-colour

Motor unit:

- 230V tubular motor; closes on battery after loss of power.

Control unit:

- Smart Control – 1 phase

Operation:

- Push button (Mounted on the unit) “up”

- Key switch (Mounted on the unit) “up-down”

Signalling:

- Connected to fire alarm centre (FAS)

- Standalone by smoke and fire detectors

Options:

- Emergency push button

- Slow-whoop and/or flashing lights (one or both sides)

Material of mounting surface:

- Steel with fire resistant cladding

- Concrete

- Stone

- Aerated concrete building blocks

Fire reaction:

B, S1, d0 (in accordance with EN 13501-1)

- Fire class B

- Smoke class S1

- Burning drops d0